Virtual Net 96 (VN96) Doc v1.0, 1996 by Hage & Steve of Wizcat

Version: 1.0, 12.03.1997

-----------------------------------------------------------------

Contents

----------

1.) Introduction

2.) General things

2.1) Devices

2.2) Installation & Handling

2.3) Do it yourself devices

3.) How to write own programs

3.1) Basics

3.2) The routine pack VN96ROUT.ASM

3.3) Asynchronous and synchronous

3.4) The first program

3.5) Synchronous programming

4.) Contact

-------------------------------------

1.) Introduction

------------------

The Virtual Net 96 is a combination of hard- and software developed by us to

connect two or more CPCs in a kind of network.

The Hardware consists of very simple devices connected to each CPC. These

devices are linked to each other with normal cables.

The software consists of utilities or games accessing a routine pack de-

veloped by us. This routine pack makes it possible to transfer data with

a speed of 60000 Baud.

This documentation introduces into the VN96 and teaches you how to write own

VN96 programs.

You can skip section 3 if you don't want to create own programs, programmers

should read the whole document.

2.) General things

--------------------

2.1) Devices

--------------

Devices and cable can be ordered from us (or buildt by yourself, see below):

* Single cable

1 cable                        1.50 DM

* Single device

1 device                       4.50 DM

* 'Add-On'

1 device and 1 cable           6.00 DM

* 'Beginner Set'

2 devices and 1 cable         10.00 DM

2.2) Installation & Handling

------------------------------

The devices are put into the printer port, the switch upwards.

Two pins at a time (one on the top of the other) belong together and must

be connected with the end of a cable. The marked wire (knot) belongs to the

upper pin. The other end of the cable has to be connected to another device.

The other two pins are used to connect a further device and so on.

If the switch is in position left the device is deactivated, in position

right it is activated.

IMPORTANT: Is the CPC switched off or if there's no VN96 program active then

the device HAS TO BE DEACTIVATED (switch in position right)! If a program

has been started you can switch on the device and the program will handle

the rest.

2.3) Do it yourself devices

-----------------------------

For users familiar with a soldering iron, here's the wiring diagram of the

devices:

---------------------o--------------------------------------- Data

<-- next CPC         |                           next CPC -->

---------------------|-----------------o--------------------- GND

|                 |

/              |

Switch  /---|           |

/                |

|                 |

Resistor       |                 |

10k          o-------+         |

| /        |         |

+-----+     |/         |         |

+-----|     |-----<          |         |

|     +-----+     |\         |         |

|                 | \--------|---------o

|                            |         |

|              Transistor    |         |

|                BC 547C     |         |

|                            |         |

______                       ____

Strobe                       Busy      GND

(Pin 1)                      (Pin 11)  (Pin 19 or another GND pin)

The pins are related to the printer port.

Our devices and cables are assembled with an unique design. To maintain

compatibility it is recommended to use the following specifications:

* The connection of a device with a cable is provided with 1.3mm pins.

There sould be four pins, two data and two GND pins.

* The data pins should be placed above, the GND pins below (to avoid

a reversed connection)

* The cables end in 1.3mm sockets and on both ends of the cable the same

of the two wires is marked with a knot.

* Switch: position right: off, position left: on

If you can't imagine how this looks like you can lend devices and wires from

us for free.

3.) How to write own programs

-------------------------------

3.1) Basics

-------------

The data wire has two states, one and zero. This state can be requested

with the 6th bit of the I/O address #F500 (Busy), for example:

IF INF(&F500) > 64 THEN PRINT "One!" ELSE PRINT "Zero!"

Or with Z80:

LD B,#F5

IN A,(C)

CP 64

JR NC,one

JR zero

This state always is equal on all connected CPCs, meaning the state is

either one or zero on ALL CPCs.

A CPC can influence this state with the 7th bit of the I/O address #EF00:

OUT &EF00,128   ' Put data wire to one

OUT &EF00,0     ' Put data wire to zero

Here an example with Z80 too:

LD BC,#EF80     ' Put data wire to one

OUT (C),C

LD B,#EF00      ' Put data wire to zero

OUT (C),A

But what happens if a CPC puts the wire to one and another puts it to zero?

Against expections the devices doesn't disintegrate or explode or something

like that.

The usual state of the data wire is one. If one or more CPCs put a zero to

the wire the whole wire switches to zero. That means zero has priority above

one, the 'ones' of other CPCs are ignored. To say it in a different way: The

wires only switches to one if ALL CPCs put a one to the wire. This is the

only way to realize a bidirectional connection. Apart from that it is very

useful for synchonisations (see below).

IMPORTANT: The first command in every VN96 program (in the BASIC loader, for

example) must put the data wire to one. After a reset the printer port puts

the data wire to zero and that would block a transmission. That's why the

device has to be switched off when there's no VN96 program running. But the

user switches on the device after he has started a VN96 program, therefore

each VN96 program has to put the data wire to one as soon as possible.

A transmission runs off like this: All CPCs willing to receive put the data

wire to one and now the CPC that wants to send can influence the wire

according to the bits he wants to transmit. But this routines must not be

written again, because there is...

3.2) The routine pack VN96ROUT.ASM

------------------------------------

The routines of the routine pack VN96ROUT.ASM (which should be on the same

disk as this doc, the newest version is available from us for free) are as

follows:

vnsnd    Transmit a byte

vnrec    Receive a byte

vnsndb   Transmit a data block

vnrecb   Receive a data block

vnzero   Wait for zero

vnone    Wait for one

vnzeri   Wait for zero without timeout

vnonei   Wait for one without timeout

vnwait   Wait a determined time

vnwais   Wait for a request routine

IMPORTANT: All routines do not change the interrupt state. But if a routine

is executed the interrupts MUST be disabled (DI).

Here the routines in detail:

Routine:       'vnsnd', transmit a byte

---------------------------------------

Jump in:       A=Byte to transmit

Jump out:      A, BC destroyed

Comments:      Byte is immediately transmitted without checking the wire.

--------------------------------------------------------------------------

Routine:       'vnrec', receive a byte

--------------------------------------

Jump in:       -

Jump out:      CY=0 --> Ok, A=received byte

CY=1 --> Timeout error

A, BC, DE destroyed

Comments:      If there's no byte received within ca. 1s the routine

returns with CY=1.

--------------------------------------------------------------------------

Routine:       'vnsndb', transmit a data block

----------------------------------------------

Jump in:       HL=Pointer to the data block

DE=Length of the data block in bytes

Jump out:      A, BC, DE, HL destroyed

Comments:      Data block is immediately transmitted without checking the

wire.

--------------------------------------------------------------------------

Routine:       'vnrecb', receive a data block

---------------------------------------------

Jump in:       HL=Pointer to a buffer

Jump out:      CY=0 --> Ok, (HL)=data block

DE=length ot the data block in bytes

CY=1 --> Timeout error

A, BC, DE, HL destroyed

Comments:      If there's no data block received within ca. 1s the routine

returns with CY=1.

--------------------------------------------------------------------------

Routine:       'vnzero', wait for zero state

--------------------------------------------

Jump in:       -

Jump out:      CY=0 --> Data wire is zero

CY=1 --> Timeout error

A, B, DE destroyed

Comments:      If the data wire doesn't switch to zero state within ca. 1s

the routine returns with CY=1.

--------------------------------------------------------------------------

Routine:       'vnone', wait for one state

------------------------------------------

Jump in:       -

Jump out:      CY=0 --> Data wire is one

CY=1 --> Timeout error

A, B, DE destroyed

Comments:      If the data wire doesn't switch to one state within ca. 1s

the routine returns with CY=1.

--------------------------------------------------------------------------

Routine:       'vnzeri', wait for zero state without timeout

------------------------------------------------------------

Jump in:       -

Jump out:      Data wire is zero

A, B, DE destroyed

Comments:      This routine returns only if the data wire is in zero

state.

--------------------------------------------------------------------------

Routine:       'vnonei', wait for one state without timeout

-----------------------------------------------------------

Jump in:       -

Jump out:      Data wire is one

A, B, DE destroyed

Comments:      This routine returns only if the data wire is in one state.

--------------------------------------------------------------------------

Routine:       'vnwait', wait a determined time

-----------------------------------------------

Jump in:       HL=Wait time in 7.63us steps

Jump out:      A, HL destroyed

Comments:      Waits HL*7.63 micro seconds.

Rule of thumb: 9 HL cycles are one raster line.

--------------------------------------------------------------------------

Routine:       'vnwais', wait for a request routine

---------------------------------------------------

Jump in:       -

Jump out:      -

Comments:      Waits the time the routines vnzero, vnone, vnzeri and

vnonei need to notice a change of the data state.

--------------------------------------------------------------------------

3.3) Asynchronous and synchronous

-----------------------------------

Gererally, two techniques are distinguished between: asynchronous and

synchronous.

Asynchrounous means simply: nothing happens - nothing is transmitted. A chat

program is an example for this. The data wire is normally in one state. If

one of the chatters presses a key, the keycode is transmitted to the other

CPCs and after this the wire swichtes to one again.

But why is the standard state one instead of zero? As sayed above: Well, if

one or more of the CPCs put the wire to zero, none of the others could

change this state any more.

Synchronous means: The connected CPCs have a determined order and data is

contiuously exchanged. Example: The Tron-like game 'Last Action Point' by

Steve of Wizcat. During the game bytes are continously exchanged between the

CPCs in a determined order. These bytes contain the joystick information of

each player. After a sychronization phase player one sends his byte, then

player two sends his byte, then player three and so on. After this all CPCs

have to calculate the exchanged data, display graphics, etc. Then the game

leads into a new sychronisation phase.

But now we have a problem: The single CPCs do not all need the same time to

process their data. That's why the transmissions have to be synchonisized.

To this matter there is a simple but important solution named synchroniza-

tion phase:

After the transmission of all bytes all CPCs put the data wire to zero and

start their calculations. If a CPC has finished this he puts the wire to

one again. But the wire won't switch to one unless ALL CPCs put the wire to

one. Therefore the CPC must wait until the wire swiches to one. All CPCs

will return synchronous from this request, when the last CPC puts the wire

to one. Then the new transmission can begin.

3.4) The first programm

-------------------------

As an example for asychronous programming a chat program was mentioned

above. Now such a chat program will be developed step by step.

First of all the wire must be put to one to avoid blocking transmissions:

ORG #A000

chatstart: LD BC,#EF80       ; put data wire to one

OUT (C),C

Now follows the main loop which checks if another CPC would like to transmit

any byte:

mainloop:  LD B,#F5          ; check data wire

IN A,(C)

CP 64             ; is it zero?

JR C,recabyte     ; yes, --> receive byte

Is the wire in zero state the program jumps to the routine which receives

the byte. If noone sends a byte, the keyboard is checked:

CALL #BB09        ; key pressed?

JR C,sndabyte     ; yes, --> transmit byte

JR mainloop

If there a key is pressed the program jumps to the transmit routine. If not

the loop starts again.

Here's the routine to receive a byte using the routine pack VN96ROUT.ASM:

recabyte:  DI                ; disable interrupts

CALL vnone        ; wait for one state

JR C,mainloop     ; timeout error, --> back to the main loop

CALL vnrec        ; receive the byte

JR C,mainloop     ; timeout error, --> back to the main loop

CALL #BB5A        ; print the char

JR mainloop       ; back to the main loop

The routine waits for the end of the start bit and then receives and prints

the byte.

Now follows the routine which transmittes a byte when a key is pressed:

sndabyte:  DI                ; disable interrupts

LD I,A            ; save the byte

LD BC,#EF00       ; send the start bit

OUT (C),C

LD HL,1500        ; wait for the others to notice it

CALL vnwait

LD BC,#EF80       ; quit the start bit

OUT (C),C

CALL vnwais       ; wait for the others to notice it

LD A,I            ; transmit the byte

CALL vnsnd

LD A,I            ; print the char

CALL #BB5A

JR mainloop       ; back to the main loop

Here the start bit and then the byte is transmitted. The first wait loop is

required because the other CPCs need a litte time to leave the main loop

(maybe in the middle of a #BB09 call). The second wait loop 'vnwais' gives

the CPCs time to return from their 'vnone' call.

3.5) Synchronous programming

------------------------------

As example for synchronous programming Steve of Wizcats 'Last Action Point'

is used here.

To use synchron programming the CPCs must have a determined order. This is

reached with a asynchronous 'chat' between the players at the beginning of

the game, the chat phase.

Every CPC calculates a random number. At the start on all CPCs the program

is loaded and the data wire is in one state. Now one player presses the

start key. This CPC transmittes a start bit (see 3.4) which starts the chat

phase. Now every CPC counts down his random number. If a CPC counts out he

transmittes a start bit followed by his player informations (name, points

etc.). The CPCs must therefore check the data state while counting down their

random numbers.

So the single CPCs one after the other transmit their data. While doing

this increasing player numbers are assigned determining the order of the

CPCs.

To quit these phase the random numbers must be restricted in its size. If

the time according to the largest possible random number is ran out the chat

phase can be quitted. No other CPC will send any data.

The chat phase is now followed by a sychnonisation phase leading into

synchonous mode (see 3.3).

4.) Contact

-------------

For free software, questions, critics, suggestions, to order devices or just

for fun write to:

* Hage of Wizcat

Abt-Rudolfstr. 46

73479 Pfahlheim

E-Mail: wsteng@rzws.fh-aalen.de

* Steve of Wizcat

Ihnbergstr. 9/1

73479 Pfahlheim

E-Mail: Christian.Stengel@T-Online.de

csteng@rzws.fh-aalen.de

* http://www.home.pages.de/~vn96